CALL CHAR

=================================================================

CALL CHAR(character-code, pattern-identifier)

=================================================================

The  CHAR  subprogram  defines  special  display characters.  The

characters are defined in a 5-by-8 grid by specifying which  dots

are 'on' and which are 'off'.  Up to seven special characters can

be  defined at one time. The characters can be displayed by using

CHR$ in a DISPLAY or PRINT statement. If a special  character  is

in  the  display  when  the  pattern  definition  is changed, the

displayed character changes immediately.

NOTE

Characters defined with the  CHAR  subprogram

are  not retained when the computer is turned

off or when the computer is placed into  CALC

mode.

'Character-code'  specifies which special display character is to

be defined.  'Character-code' must be a value from 0 through 6.

'Pattern-identifier' is  a  string  expression  whose  value

defines the pattern for one or more special display characters.

0   The  first  16  characters  of 'pattern-identifier define the

specified 'character-code'.  If 'pattern-identifier' is  less

than  16  characters, the remaining characters are considered

to be zeros.

0   If 'pattern-identifier' is greater than  16  characters,  the

extra characters define the next sequential 'character-code',

until  all  the  'pattern-identifier'  characters  have  been

assigned to a 'character-code'.

0   If 'pattern-identifier' has enough extra characters to define

past 'character-code' 6, the extra characters are ignored.

0   If 'pattern-identifier' is a null string, an error occurs.

Each  pair  of  characters in 'pattern-identifier' describes

the pattern in one row of the grid.  The left character is  1  or

0,  indicating  that  the  left block is on or off, respectively.

The right character is a hexadecimal digit (0  through  F)  whose

binary equivalent is used to determine which dots are on and off,

as  described  above.  The rows are described from top to bottom.

Note that there is a slight break in the display between the  top

seven rows and the eighth row.

The following table shows all possible on/off conditions for

each   row,  and  the  binary  and  hexadecimal  codes  for  each

condition.

Binary Code   Hexadecimal

Dot Pattern   0=off:1=on       Code

-----------   -----------   -----------

| | | | | |      00000          00

| | | | |*|      00001          01

| | | |*| |      00010          02

| | | |*|*|      00011          03

| | |*| | |      00100          04

| | |*| |*|      00101          05

| | |*|*| |      00110          06

| | |*|*|*|      00111          07

| |*| | | |      01000          08

| |*| | |*|      01001          09

| |*| |*| |      01010          0A

| |*| |*|*|      01011          0B

| |*|*| | |      01100          0C

| |*|*| |*|      01101          0D

| |*|*|*| |      01110          0E

| |*|*|*|*|      01111          0F

|*| | | | |      10000          10

|*| | | |*|      10001          11

|*| | |*| |      10010          12

|*| | |*|*|      10011          13

|*| |*| | |      10100          14

|*| |*| |*|      10101          15

|*| |*|*| |      10110          16

|*| |*|*|*|      10111          17

|*|*| | | |      11000          18

|*|*| | |*|      11001          19

|*|*| |*| |      11010          1A

|*|*| |*|*|      11011          1B

|*|*|*| | |      11100          1C

|*|*|*| |*|      11101          1D

|*|*|*|*| |      11110          1E

|*|*|*|*|*|      11111          1F

To define the dot pattern  pictured  below,  type  the  following

line:

CALL CHAR(0,"04150E04040E1504")

l|r r r r        [l)eft or r)ight blocks]

__|_______    Hex codes

row 1|_|_|*|_|_|    04

row 2|*|_|*|_|*|    15

row 3|_|*|*|*|_|    0E

row 4|_|_|*|_|_|    04

row 5|_|_|*|_|_|    04

row 6|_|*|*|*|_|    0E

row 7|*|_|*|_|*|    15

row 8|_|_|*|_|_|    04

To  display  the  special  character  defined above, enter 'PRINT

CHR$(0)'.  Note that the underline cursor  also  appears  in  the

display in this case.