CALL IOX

=================================================================

/(device, command [, status-variable])\

CALL IOX {                                         }

\(string-variable [, status-variable]) /

=================================================================

The IOX subprogram is an extended version of  the  IO  subprogram

built  into  the  TI-74.   It performs special control operations

which are not available in the TI-74 BASIC, but may be  supported

by  some  peripherals.   Proper  use  of this subprogram requires

knowledge of input/output  (I/O)  data  structures  and  specific

peripheral capabilities. Refer to the peripheral manuals for more

information regarding these capabilities.

'Device' is the number associated with the peripheral device

and  can  be  from  1  through  255.   If device is zero (0), all

peripherals attached will receive and act on the command sent.

Depending on the command used with device zero, the I/O  bus

may  become  'locked  up' with too many peripherals attempting to

communicate at the same time.  The bus reset command,  value  255

decimal,  may  be  specified  with  device  zero  and not cause a

problem.  No peripherals respond with a return message  for  this

command.

'Command'  is a numeric code that specifies the operation to

be performed by the device.

'String-variable' contains  from  2  through  12  characters

which  represent  the  data  required for the I/O operation.  The

data passed to the  IOX  subprogram  are  interpreted  as  binary

values.    The  'string-variable'  is  always  returned  with  12

characters.  The data length and status  may  be  modified.   The

format of the string is shown in the following table:

Field          Field

Name           Length    Description

-------------  -------   ---------------------------------

device           1       peripheral device code

command          1       operation command code

file number      1       file number as assigned in BASIC

record number    2       record number within a file

buffer length    2       size of the buffer for data

received from the peripheral

data length      2       number of characters to be sent

to the peripheral

status           1       status code returned by device

buffer pointer   2       highest address of data buffer

Specific  requirements for this data are given in the peripherals

manuals.

'Status-variable' is an optional numeric variable  in  which

information  regarding the result of the operation is stored.  If

no I/O error occurred, 'status-variable'  is  zero.   If  an  I/O

error  occurred, 'status-variable' contains the corresponding I/O

error code.  The inclusion of  a  'status-variable'  affects  the

computer's response to the occurrence of an I/O error.  If an I/O

error occurs when 'status-variable' is given, no error message is

displayed  and  the  error  cannot be handled by ON ERROR.  If an

error occurs when 'status-variable' is omitted,  the  message  is

displayed or the error can be handled by ON ERROR.

140 CALL IOX(45,1)

The  above  example  closes  device 45. (A command code of 1 is a

CLOSE operation.)